Cisco Packet Tracer

Routing, NAT, IPv6 and ACL Lab Manual

Five assignments — every command, every click, every check

Static routing, OSPF, NAT/PAT overload, IPv6 addressing and routing, and extended access control lists. Each section is self-contained: build the topology, run the configuration, verify, then read the concept notes.

## How to read this document

- Numbered "Step" headings tell you what to do, in order.

- A grey "Where:" line tells you exactly which tab or panel inside Packet Tracer to use.

- Monospaced boxes are commands to type verbatim into a device's CLI tab, or into a PC's Command Prompt.

- Green TIP boxes are shortcuts and habits worth building. Orange WATCH OUT boxes are mistakes that are easy to make and hard to spot.

## Before you start — three things that break every routing lab

1. Serial interfaces do not exist until you add the module

Assignments 1 and 2 use serial links (S0/0/0). Modern ISR routers ship with no serial ports at all, so "interface Serial0/0/0" is rejected as an invalid interface until you physically fit the card.

Where: Double-click the router → Physical tab → click the power switch on the chassis to turn it OFF → drag an HWIC-2T module from the left list into an empty slot → turn the power back ON.

WATCH OUT — The router MUST be powered off before you add or remove a module. Dragging a card into a running router either refuses or silently fails, and you are then left configuring an interface that does not exist.

TIP — Fit the module on BOTH routers before you cable them together. A serial cable cannot be attached to a router with no serial port, and Packet Tracer will simply refuse the connection with no useful explanation.

2. The DCE end of a serial link needs a clock rate

A serial link has a DCE end and a DTE end. The DCE end provides timing, and without it the line protocol never comes up — the interface reports "up, line protocol down", which looks exactly like a cabling fault.

! on the DCE end only interface Serial0/0/0 clock rate 64000 no shutdown

TIP — Find out which end is DCE with "show controllers Serial0/0/0". The output says either "DCE" or "DTE" near the top. In Packet Tracer, the DCE end is the one where you started drawing the cable, and the canvas shows a small clock icon on that side.

3. Interface names differ by router model

Depending on which router you drag out, the same physical port may be called GigabitEthernet0/0, GigabitEthernet0/0/0, or FastEthernet0/0. Never type an interface name from a diagram without checking.

enable show ip interface brief

Use the exact names this command prints. Everything else in this document assumes you have done that substitution.

## Universal habits

- Every router interface starts shut down. "no shutdown" is not optional — it is the single most common reason a correctly addressed link stays dead.

- Read the prompt before every command. Router(config)# is global config; Router(config-if)# is interface config; Router(config-router)# is routing-protocol config. A command rejected as invalid input is usually a correct command typed in the wrong mode.

- Finish every device with "write memory". Configuration lives in RAM until you do, and is lost on reload.

- Verify after each stage rather than at the end. Finding one broken link early is easy; finding it inside a finished four-router topology is not.

# 1 — Static Routing Between Two LANs

Objective: connect two separate LANs across a serial WAN link using manually configured routes, and verify end-to-end connectivity.

## Topology and addressing

LAN-A (192.168.1.0/24) hangs off R1 via SW1. LAN-B (192.168.2.0/24) hangs off R2 via SW2. The two routers are joined by a serial link on the 10.0.0.0/30 network.

| Device | Interface | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| R1 | G0/0 | 192.168.1.1 | 255.255.255.0 | — |
| R1 | S0/0/0 | 10.0.0.1 | 255.255.255.252 | — |
| R2 | S0/0/0 | 10.0.0.2 | 255.255.255.252 | — |
| R2 | G0/0 | 192.168.2.1 | 255.255.255.0 | — |
| PC-A1 | FastEthernet0 | 192.168.1.10 | 255.255.255.0 | 192.168.1.1 |
| PC-A2 | FastEthernet0 | 192.168.1.11 | 255.255.255.0 | 192.168.1.1 |
| PC-B1 | FastEthernet0 | 192.168.2.10 | 255.255.255.0 | 192.168.2.1 |
| PC-B2 | FastEthernet0 | 192.168.2.11 | 255.255.255.0 | 192.168.2.1 |

TIP — A /30 mask (255.255.255.252) gives exactly two usable addresses — .1 and .2 — which is precisely what a point-to-point link needs and nothing more. This is standard practice for WAN links and marks you out as knowing why you chose it.

STEP 1   Place and cable

Where: Network Devices → Routers → drag two routers. Network Devices → Switches → two 2960 switches. End Devices → four PCs. Connections → Automatic for the LAN links; Connections → Serial DCE for the router-to-router link.

- PC-A1, PC-A2 → SW1;  SW1 → R1 G0/0

- PC-B1, PC-B2 → SW2;  SW2 → R2 G0/0

- R1 S0/0/0 → R2 S0/0/0 (serial cable)

WATCH OUT — Fit an HWIC-2T serial module in BOTH routers first (Physical tab, power off). Without it there is no serial port to plug into and the cable is refused.

STEP 2   Configure R1

Where: R1 → CLI tab.

enable configure terminal hostname R1 interface GigabitEthernet0/0 ip address 192.168.1.1 255.255.255.0 no shutdown exit interface Serial0/0/0 ip address 10.0.0.1 255.255.255.252 clock rate 64000 no shutdown exit end

STEP 3   Configure R2

Where: R2 → CLI tab.

enable configure terminal hostname R2 interface GigabitEthernet0/0 ip address 192.168.2.1 255.255.255.0 no shutdown exit interface Serial0/0/0 ip address 10.0.0.2 255.255.255.252 no shutdown exit end

TIP — Only one end gets "clock rate" — the DCE end. Adding it to the DTE end is harmless but does nothing; omitting it from the DCE end leaves the link permanently down.

STEP 4   Add the static routes

Where: Each router → CLI tab.

Each router already knows about its own directly connected networks. What it does not know is how to reach the network on the far side. One static route each fixes that.

! --- on R1: to reach LAN-B, send traffic to R2 --- configure terminal ip route 192.168.2.0 255.255.255.0 10.0.0.2 end write memory

! --- on R2: to reach LAN-A, send traffic to R1 --- configure terminal ip route 192.168.1.0 255.255.255.0 10.0.0.1 end write memory

The syntax reads: to reach network <destination> with mask <mask>, hand the packet to <next hop>. The next hop is always an address on a network you are already directly connected to — never the far-side LAN address.

WATCH OUT — Routing must work in BOTH directions. A very common mistake is configuring only R1 and then wondering why pings fail — the request reaches LAN-B fine, but R2 has no route back and the reply is dropped. Every ping needs a path there and a path home.

STEP 5   Set the PC addressing

Where: Each PC → Desktop tab → IP Configuration → Static.

Enter the IP, mask and default gateway from the addressing table. The gateway is the router interface on that PC's own LAN — 192.168.1.1 for the LAN-A PCs, 192.168.2.1 for the LAN-B PCs.

WATCH OUT — A PC with no default gateway can reach its own LAN perfectly but nothing beyond it. If local pings work and remote pings fail, check the gateway field before you touch any router configuration.

STEP 6   Verify

Where: R1 and R2 → CLI tab; then PCs → Desktop → Command Prompt.

! on each router show ip interface brief ! G0/0 and S0/0/0 both up/up show ip route ! look for the S entry you added

! from PC-A1 ping 192.168.1.1 ! own gateway ping 10.0.0.2 ! far side of the serial link ping 192.168.2.10 ! PC-B1, the real test tracert 192.168.2.10 ! shows the hop-by-hop path

In "show ip route", directly connected networks are marked C and your static routes are marked S. If an S entry is missing, the route was never accepted — re-enter it and watch for an error.

TIP — tracert is the command that earns marks in a viva. It prints each router the packet passes through, proving the traffic really is crossing the WAN link rather than being answered locally.

# 2 — Dynamic Routing using OSPF

Objective: replace manual routes with OSPF so that the routers discover each other and build their routing tables automatically.

## Topology and addressing

Three routers in a triangle. R2 sits at the top with serial links down to R1 and R3; R1 and R3 are also joined directly. Each router has its own LAN.

| Device | Interface | IP Address | Subnet Mask | Connects to |
| --- | --- | --- | --- | --- |
| R1 | G0/0 | 192.168.1.1 | 255.255.255.0 | LAN with PC1, PC2 |
| R1 | S0/0/0 | 10.0.12.1 | 255.255.255.252 | R2 |
| R1 | S0/0/1 | 10.0.13.1 | 255.255.255.252 | R3 |
| R2 | S0/0/0 | 10.0.12.2 | 255.255.255.252 | R1 |
| R2 | S0/0/1 | 10.0.23.1 | 255.255.255.252 | R3 |
| R2 | G0/0 | 192.168.2.1 | 255.255.255.0 | its own LAN |
| R3 | S0/0/1 | 10.0.23.2 | 255.255.255.252 | R2 |
| R3 | S0/0/0 | 10.0.13.2 | 255.255.255.252 | R1 |
| R3 | G0/0 | 192.168.3.1 | 255.255.255.0 | LAN with PC3, PC4 |

| PC | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- |
| PC1 | 192.168.1.10 | 255.255.255.0 | 192.168.1.1 |
| PC2 | 192.168.1.11 | 255.255.255.0 | 192.168.1.1 |
| PC3 | 192.168.3.10 | 255.255.255.0 | 192.168.3.1 |
| PC4 | 192.168.3.11 | 255.255.255.0 | 192.168.3.1 |

WATCH OUT — The R1-to-R3 link is drawn as 19.0.13.0/30 on some versions of the diagram. That is almost certainly a typo for 10.0.13.0/30, which is consistent with the other two WAN links. Use 10.0.13.0/30 unless your instructor says otherwise — but whichever you pick, use it consistently on both routers and in the OSPF network statement.

STEP 1   Address every interface first

Where: Each router → CLI tab.

OSPF cannot advertise a network that is not yet configured and up. Address and enable all interfaces before you start the routing protocol.

! --- R1 --- enable configure terminal hostname R1 interface GigabitEthernet0/0 ip address 192.168.1.1 255.255.255.0 no shutdown exit interface Serial0/0/0 ip address 10.0.12.1 255.255.255.252 clock rate 64000 no shutdown exit interface Serial0/0/1 ip address 10.0.13.1 255.255.255.252 clock rate 64000 no shutdown exit end

Repeat the same pattern on R2 and R3 with their own addresses from the table. Set "clock rate" only on whichever end of each link is the DCE side.

STEP 2   Enable OSPF on R1

Where: R1 → CLI tab.

configure terminal router ospf 1 network 192.168.1.0 0.0.0.255 area 0 network 10.0.12.0 0.0.0.3 area 0 network 10.0.13.0 0.0.0.3 area 0 exit end write memory

STEP 3   Enable OSPF on R2 and R3

Where: R2 → CLI tab, then R3 → CLI tab.

! --- R2 --- configure terminal router ospf 1 network 192.168.2.0 0.0.0.255 area 0 network 10.0.12.0 0.0.0.3 area 0 network 10.0.23.0 0.0.0.3 area 0 exit end write memory

! --- R3 --- configure terminal router ospf 1 network 192.168.3.0 0.0.0.255 area 0 network 10.0.13.0 0.0.0.3 area 0 network 10.0.23.0 0.0.0.3 area 0 exit end write memory

## Understanding the network statement

The line "network 192.168.1.0 0.0.0.255 area 0" does not advertise a network directly. It says: any interface on this router whose address falls inside 192.168.1.0/24 should participate in OSPF, in area 0.

| Subnet mask | Wildcard mask | Meaning |
| --- | --- | --- |
| 255.255.255.0 (/24) | 0.0.0.255 | Match the first three octets |
| 255.255.255.252 (/30) | 0.0.0.3 | Match all but the last two bits |
| 255.255.0.0 (/16) | 0.0.255.255 | Match the first two octets |

TIP — A wildcard mask is the subnet mask inverted: subtract each octet from 255. So /24 (255.255.255.0) becomes 0.0.0.255, and /30 (255.255.255.252) becomes 0.0.0.3. Getting this backwards is the classic OSPF mistake — a subnet mask in a network statement silently matches nothing, so the adjacency never forms and no error is printed.

WATCH OUT — "router ospf 1" — the 1 is a process ID that is only locally significant. It does NOT have to match on other routers. The "area 0" DOES have to match across all of them. Students routinely get this the wrong way round.

STEP 4   Set the PC addressing

Where: Each PC → Desktop tab → IP Configuration → Static.

Use the PC table above. The gateway is the G0/0 address of that PC's own router.

STEP 5   Verify

Where: Each router → CLI tab; then PCs → Desktop → Command Prompt.

show ip route ! OSPF routes appear with an O show ip ospf neighbor ! every directly connected router listed, state FULL show ip protocols ! confirms OSPF is running and which networks it covers

! from PC1 ping 192.168.3.10 ! PC3, two hops away tracert 192.168.3.10 ! shows which path OSPF chose

In "show ip route", routes learned through OSPF are marked O. Seeing an O route to 192.168.3.0 on R1 proves the protocol worked — R1 was never told about that network, it discovered it.

TIP — OSPF takes 30 to 40 seconds to converge after you enable it. If "show ip ospf neighbor" is empty immediately after configuration, wait and run it again before assuming something is wrong.

WATCH OUT — If a neighbour never appears: check that both ends of the link are up/up, that the area number matches, and that your wildcard masks are inverted correctly. Those three account for almost every failed adjacency.

# 3 — NAT / PAT (Overload)

Objective: let a whole private network share one public address to reach an outside web server, and inspect the translation table that makes it work.

## Topology and addressing

| Device | Interface | IP Address | Subnet Mask | Role |
| --- | --- | --- | --- | --- |
| R1 | G0/0/0 | 192.168.10.1 | 255.255.255.0 | Inside (private) |
| R1 | G0/0/1 | 203.0.113.2 | 255.255.255.0 | Outside (public) |
| PC1 | FastEthernet0 | 192.168.10.10 | 255.255.255.0 | gw 192.168.10.1 |
| PC2 | FastEthernet0 | 192.168.10.11 | 255.255.255.0 | gw 192.168.10.1 |
| Web Server | FastEthernet0 | 203.0.113.10 | 255.255.255.0 | gw 203.0.113.2 |

TIP — 192.168.x.x is private address space — it cannot be routed on the public internet, which is exactly why translation is needed. 203.0.113.x is a documentation range reserved for examples like this one.

WATCH OUT — Interface names shown are for an ISR 4321 (GigabitEthernet0/0/0 and 0/0/1). Confirm with "show ip interface brief" on your own router before typing anything — this has differed from the diagram every time so far in this lab.

TIP — Skip a Cloud-PT for this assignment. It adds nothing here and the coaxial/port-mapping issues from the Cellular Network assignment are not worth repeating — a direct copper cable from R1's outside interface to the Web Server is all this design needs.

STEP 1   Place and cable

Where: Two PCs → a switch → R1's inside interface. R1's outside interface → the Web Server directly. Connections → Automatic throughout.

STEP 2   Address the interfaces

Where: R1 → CLI tab. Substitute the real interface names from "show ip interface brief".

enable configure terminal hostname R1 interface GigabitEthernet0/0/0 ip address 192.168.10.1 255.255.255.0 no shutdown exit interface GigabitEthernet0/0/1 ip address 203.0.113.2 255.255.255.0 no shutdown exit end

STEP 3   Mark which interface is inside and which is outside

Where: R1 → CLI tab.

configure terminal interface GigabitEthernet0/0/0 ip nat inside exit interface GigabitEthernet0/0/1 ip nat outside exit end

WATCH OUT — Without these two lines nothing is translated, no error appears, and pings from PC1 simply fail. This is the most commonly forgotten step in the whole assignment. NAT has no idea which direction is which until you tell it.

TIP — "show running-config interface <name>" is rejected as invalid input in Packet Tracer's IOS — that specific scoped form isn't supported. Use "show running-config ¦ include ip nat" instead to see just the NAT-related lines across the whole config; piping does work.

STEP 4   Define which addresses may be translated

Where: R1 → CLI tab.

configure terminal access-list 1 permit 192.168.10.0 0.0.0.255 end

This standard ACL is not filtering anything. It is simply a list that says "these are the inside addresses eligible for translation".

STEP 5   Turn on PAT (NAT overload)

Where: R1 → CLI tab.

configure terminal ip nat inside source list 1 interface GigabitEthernet0/0/1 overload end write memory

Read it as: translate inside sources matching list 1, using the address of the outside interface, and overload that single address across many hosts by tracking port numbers.

TIP — "overload" is the keyword that makes this PAT rather than plain NAT. Without it, one inside host would consume one public address and the second host would get nothing. With it, hundreds of hosts share one public address because each conversation is distinguished by its source port number.

STEP 6   Add a default route towards the ISP

Where: R1 → CLI tab.

configure terminal ip route 0.0.0.0 0.0.0.0 GigabitEthernet0/0/1 end write memory

0.0.0.0 0.0.0.0 means "any destination I have no better route for". It is the catch-all that sends unknown traffic — everything destined for the outside world — out of the public-facing interface.

STEP 7   Address the PCs and the server

Where: PCs → Desktop tab → IP Configuration. Web Server → Desktop tab → IP Configuration, THEN Services tab → HTTP.

Use the addressing table. On the web server, set its static IP on the Desktop tab first, then open the Services tab, select HTTP, and switch the service On.

WATCH OUT — CONFIRMED IN LAB — this is the step most likely to get silently skipped, and the failure it produces is confusing because everything else tests as working. It is easy to open the server, go straight to the Services tab, switch HTTP on, and never visit the Desktop tab at all — leaving the server with no IP address. The symptom: the browser says "Request Timeout", but "show ip nat translations" on R1 shows real TCP entries on port 80 — the request genuinely left the router and arrived somewhere. That combination means the packets are correct and nothing is listening at the destination to answer them. Always confirm the server's own Desktop → IP Configuration before chasing the HTTP toggle or the NAT config any further.

WATCH OUT — On any server, the Services tab has a small On/Off radio pair near the top-left of the service panel. It is easy to miss against the dark interface. If HTTP is Off, nothing else on that page matters — check that radio FIRST when a service seems dead. The HTTP page actually has TWO groups side by side, HTTP on the left and HTTPS on the right, each reading On then Off left to right — only the left group matters here. Click the radio, then click elsewhere on the panel before closing the window, so the change actually commits.

STEP 8   Test and inspect the translations

Where: PC1 → Desktop → Command Prompt, then Desktop → Web Browser. Then R1 → CLI tab.

! from PC1 ping 203.0.113.10

Then open PC1 → Desktop → Web Browser and enter http://203.0.113.10 — the server's default page should load.

! on R1, immediately afterwards show ip nat translations show ip nat statistics

"show ip nat translations" is the evidence. Each row shows an Inside local address (the PC's real private address plus port) alongside an Inside global address (the shared public address plus a different port). Generate traffic from PC2 as well and you will see two rows sharing one public address but with different port numbers — that is PAT working.

TIP — Use this command as a diagnostic FIRST, not just a final check. If a ping or the browser test fails, run this before troubleshooting anything else. Seeing entries here — even from a "failed" ping or a browser that timed out — proves the entire chain up through NAT (routing, ACL, inside/outside marking, overload) is working correctly, and points the problem at the destination itself rather than at R1. That is exactly how the missing server IP address above gets diagnosed: real translation entries exist, but nothing answers.

WATCH OUT — Translations time out after a period of inactivity. If the table is empty, it is usually because you waited too long after the ping. Send fresh traffic and check again immediately.

# 4 — IPv6 Addressing and Routing

Objective: configure IPv6 on two routers and their LANs, enable IPv6 routing, and achieve end-to-end IPv6 connectivity.

## Topology and addressing

| Device | Interface | IPv6 Address | Notes |
| --- | --- | --- | --- |
| R1 | G0/0/0 | 2001:DB8:1:1::1/64 | LAN-1 gateway |
| R1 | S0/1/0 | 2001:DB8:0:0::1/64 | link to R2 |
| R2 | S0/1/0 | 2001:DB8:0:0::2/64 | link to R1 |
| R2 | G0/0/0 | 2001:DB8:2:1::1/64 | LAN-2 gateway |
| PC1 | FastEthernet0 | 2001:DB8:1:1::10/64 | gw 2001:DB8:1:1::1 |
| PC2 | FastEthernet0 | 2001:DB8:1:1::11/64 | gw 2001:DB8:1:1::1 |
| PC3 | FastEthernet0 | 2001:DB8:2:1::10/64 | gw 2001:DB8:2:1::1 |
| PC4 | FastEthernet0 | 2001:DB8:2:1::11/64 | gw 2001:DB8:2:1::1 |

TIP — The :: in an IPv6 address means "one or more groups of zeros, compressed". 2001:DB8:1:1::1 is the full address 2001:0DB8:0001:0001:0000:0000:0000:0001 written short. You may use :: only once per address, because otherwise it would be ambiguous how many zeros belong on each side.

WATCH OUT — READ THIS PREFIX CAREFULLY: it is 2001:DB8 — D, B, eight. Not 2001:D88 (D, eight, eight). These look nearly identical on screen and are completely different networks. Confirmed in lab: the routers were configured with D88 while the PCs correctly used DB8, and the result was that every routing table looked flawless, both static routes were present, all interfaces were up, and absolutely nothing pinged — because the PCs were not on the same network as their own gateways. 2001:DB8::/32 is the officially reserved IPv6 documentation prefix, which is why every textbook uses it. After configuring everything, read every prefix in "show ipv6 route" character by character before troubleshooting anything else.

WATCH OUT — The router-to-router link must be SERIAL, not Gigabit. It is easy to cable the two routers together using their spare Gig ports — the link goes green immediately and looks entirely correct, which makes the mistake convincing. But the exercise specifies a serial WAN link, and using Gig quietly removes the DCE/DTE and clock-rate concepts the assignment exists to teach. Fit a NIM-2T in both routers and use the explicit Serial DCE cable.

STEP 1   Enable IPv6 routing — do this first

Where: R1 → CLI tab, then R2 → CLI tab.

enable configure terminal ipv6 unicast-routing end

WATCH OUT — This single line is the one that catches everyone. Without it a router will happily accept IPv6 addresses on its interfaces and will answer pings to itself, but it will not forward a single IPv6 packet between networks. Everything looks configured and nothing works. Type it on both routers before anything else.

STEP 2   Address R1

Where: R1 → CLI tab. Confirm real interface names with "show ip interface brief" first.

configure terminal hostname R1 interface GigabitEthernet0/0/0 ipv6 address 2001:DB8:1:1::1/64 no shutdown exit interface Serial0/1/0 ipv6 address 2001:DB8:0:0::1/64 clock rate 64000 no shutdown exit end

WATCH OUT — "show ip interface brief" will report every interface as "unassigned" throughout this assignment. That is normal, not a fault — that command only reports IPv4 addressing, and this is a pure IPv6 build. Use it to discover interface names and check up/down status; use "show ipv6 interface brief" to actually see your addresses.

STEP 3   Address R2

Where: R2 → CLI tab. R2's interface names are NOT guaranteed to match R1's — check separately.

configure terminal hostname R2 interface GigabitEthernet0/0/0 ipv6 address 2001:DB8:2:1::1/64 no shutdown exit interface Serial0/1/0 ipv6 address 2001:DB8:0:0::2/64 no shutdown exit end

TIP — Note the command is "ipv6 address", and the prefix length is written attached to the address with a slash — not as a separate mask argument the way IPv4 does it.

WATCH OUT — An IPv6 interface can hold MULTIPLE addresses at once, so "ipv6 address" ADDS rather than replaces — a real difference from IPv4, where retyping "ip address" simply overwrites. To correct a mistyped address you must remove the old one explicitly: "no ipv6 address 2001:D88:1:1::1/64" before adding the right one. Skip that and both prefixes stay live on the interface simultaneously. The same applies to static routes — "no ipv6 route <old-prefix> <old-next-hop>" before adding the corrected line.

WATCH OUT — "administratively down" and plain "down/down" mean different things. Administratively down = missing "no shutdown", a config problem. Plain down/down = the interface is enabled but sees no partner, a cabling problem — either the cable was never drawn, or the far end is not configured yet. Confirmed in lab: R2's LAN interface read down/down purely because the cable to its switch had never been drawn.

WATCH OUT — A router can have a NIM-2T fitted with NO cable attached, and "show ip interface brief" will not reveal that — an uncabled port and a port whose far end is unconfigured both read down/down. To tell them apart run "show controllers serial 0/1/0"; if it reports "No serial cable attached", the port is genuinely unconnected and no configuration will help. Note the command needs a space — "show controllers serial 0/1/0", not "show controllers Serial0/1/0", which is rejected as invalid input.

STEP 4   Add the IPv6 static routes

Where: Each router → CLI tab.

! --- on R1: reach LAN-2 via R2 --- configure terminal ipv6 route 2001:DB8:2:1::/64 2001:DB8:0:0::2 end write memory

! --- on R2: reach LAN-1 via R1 --- configure terminal ipv6 route 2001:DB8:1:1::/64 2001:DB8:0:0::1 end write memory

The structure mirrors IPv4 static routing exactly: destination prefix, then the next-hop address. Only the address format and the keyword change.

WATCH OUT — BOTH routers need their route — check both, not just one. The exact failure seen in lab: R1's routing table had 6 entries including its S route, while R2's had only 5 with no S line at all. R1 could reach R2's LAN perfectly, but R2 had no route home, so every ping timed out. Same lesson as static routing in Assignment 1 — a ping needs a path there AND a path back. Run "show ipv6 route" on both and compare the entry counts; with both static routes in place each router shows 6 entries. If one has fewer, that is the router missing its route.

STEP 5   Address the PCs

Where: Each PC → Desktop tab → IP Configuration → the IPv6 Configuration section → Static.

Enter the IPv6 address with its /64 prefix length and the IPv6 gateway from the table. The IPv6 fields sit below the IPv4 fields on the same panel — scroll down if you do not see them.

TIP — Leave the IPv4 fields blank or as they are. This assignment is pure IPv6, and a PC can hold both address families at once without conflict.

STEP 6   Verify

Where: Routers → CLI tab; PCs → Desktop → Command Prompt.

! on each router show ipv6 interface brief ! interfaces up with their addresses show ipv6 route ! C for connected, S for your static routes

! from PC1 ping 2001:DB8:1:1::1 ! own gateway ping 2001:DB8:2:1::10 ! PC3, across the WAN link

Every interface will also show a link-local address beginning FE80::, which appears automatically and is never configured by hand. It is used for router-to-router communication on the local segment and its presence is normal.

WATCH OUT — If pings to your own gateway work but pings across the link fail, the cause is almost always the missing "ipv6 unicast-routing" from Step 1, or a static route that was never added. Check both before suspecting the addressing.

# 5 — Access Control Lists

Objective: use an extended ACL to stop the Students LAN reaching the Server LAN while leaving the Faculty LAN's access intact and all other traffic flowing.

## Topology and addressing

One router, R1, with three LANs hanging off it — Students, Faculty and Servers. Every LAN reaches the others through R1, which is exactly why R1 is the right place to enforce the policy.

| Device | Interface | IP Address | Subnet Mask | Network |
| --- | --- | --- | --- | --- |
| R1 | G0/0 | 192.168.10.1 | 255.255.255.0 | Students LAN |
| R1 | G0/1 | 192.168.20.1 | 255.255.255.0 | Faculty LAN |
| R1 | G0/2 | 192.168.30.1 | 255.255.255.0 | Server LAN |
| PC-S1 | FastEthernet0 | 192.168.10.10 | 255.255.255.0 | gw 192.168.10.1 |
| PC-S2 | FastEthernet0 | 192.168.10.11 | 255.255.255.0 | gw 192.168.10.1 |
| PC-F1 | FastEthernet0 | 192.168.20.10 | 255.255.255.0 | gw 192.168.20.1 |
| Server | FastEthernet0 | 192.168.30.10 | 255.255.255.0 | gw 192.168.30.1 |

WATCH OUT — This needs three LAN interfaces on one router. If your router model only has two, open its Physical tab, power it off, and fit an additional Ethernet module (HWIC-1GE-SFP or similar) before you start. Check with "show ip interface brief" what you actually have.

STEP 1   Cable and address everything

Where: R1 → CLI tab, then each PC and the Server → Desktop tab → IP Configuration.

enable configure terminal hostname R1 interface GigabitEthernet0/0 ip address 192.168.10.1 255.255.255.0 no shutdown exit interface GigabitEthernet0/1 ip address 192.168.20.1 255.255.255.0 no shutdown exit interface GigabitEthernet0/2 ip address 192.168.30.1 255.255.255.0 no shutdown exit end write memory

STEP 2   Verify BEFORE writing any ACL

Where: PC-S1 and PC-F1 → Desktop → Command Prompt.

! from PC-S1 (Students) ping 192.168.30.10 ! must SUCCEED at this stage ! from PC-F1 (Faculty) ping 192.168.30.10 ! must SUCCEED at this stage

WATCH OUT — Both must pass before you add the ACL. If you apply filtering on top of a network that was already broken, a failed ping tells you nothing about which of the two layers caused it — and debugging both at once is how people lose an entire lab session.

STEP 3   Write the extended ACL

Where: R1 → CLI tab.

configure terminal ! 1. block Students from the Server LAN access-list 110 deny ip 192.168.10.0 0.0.0.255 192.168.30.0 0.0.0.255 ! 2. everything else is allowed -- including Faculty to Server, ! and Students to anywhere that is not the Server LAN access-list 110 permit ip any any end

The Faculty LAN needs no rule of its own. It is permitted by the second line, because the deny above it only matches traffic sourced from 192.168.10.0/24.

STEP 4   Apply it to the right interface, in the right direction

Where: R1 → CLI tab.

configure terminal interface GigabitEthernet0/0 ip access-group 110 in exit end write memory

Applied inbound on G0/0 — the interface facing the Students LAN. The router inspects each packet as it arrives from the students, before doing any routing work.

TIP — The rule of thumb: place an extended ACL as close to the SOURCE of the traffic as possible. It drops unwanted packets at the first hop instead of carrying them across the network only to discard them at the far end.

## Three rules that decide whether an ACL works

| Rule | Why it matters |
| --- | --- |
| Order is everything | An ACL is read top to bottom and stops at the first match. A broad permit placed above your deny makes the deny unreachable — it will never fire. |
| There is an invisible deny at the end | Every ACL finishes with an implicit "deny any any" that you cannot see. Without an explicit "permit ip any any" as the last line, the ACL blocks everything, not just your target. |
| Direction and placement | "in" filters traffic arriving on the interface, "out" filters traffic leaving it. The same ACL applied in the wrong direction, or on the wrong interface, silently does nothing useful. |

WATCH OUT — ACL lines APPEND — they do not overwrite. Retyping a corrected line adds it to the bottom of the list, below the wrong one, where it may never be reached. To fix a mistake you must remove the entire list with "no access-list 110" and retype it from scratch.

STEP 5   Prove the policy works

Where: PC-S1 and PC-F1 → Desktop → Command Prompt.

| Run from | Command | Required result | What it proves |
| --- | --- | --- | --- |
| PC-S1 (Students) | ping 192.168.30.10 | FAILS | Students blocked from the Server LAN |
| PC-S1 (Students) | ping 192.168.20.10 | SUCCEEDS | Students can still reach Faculty — the block is selective |
| PC-F1 (Faculty) | ping 192.168.30.10 | SUCCEEDS | Faculty access to the server is untouched |

A blocked ping returns "Destination host unreachable" from 192.168.10.1 — the router itself refusing to forward. That reply coming from your own gateway is the signature of an ACL drop rather than a routing failure.

STEP 6   Show the evidence

Where: R1 → CLI tab.

show access-lists show ip interface GigabitEthernet0/0 ! confirms which ACL is applied, and its direction

"show access-lists" prints match counters beside each line. A rising count on the deny line proves the rule is actually being hit, rather than the ping failing for some unrelated reason. Screenshot this next to your ping results.

# Troubleshooting Reference

| Symptom | Most likely cause and fix |
| --- | --- |
| %Invalid interface type and number | Wrong name for this router model, or the module is not fitted. Run "show ip interface brief" and use the exact name listed. Serial ports need an HWIC-2T added in the Physical tab with the router powered off. |
| Serial interface: up, line protocol down | The DCE end has no clock rate. Find the DCE end with "show controllers Serial0/0/0" and add "clock rate 64000" there. |
| Interface administratively down | Missing "no shutdown". Enter interface config mode and issue it. |
| Ping works one way but not back | The return route is missing. Static routing must be configured on BOTH routers — every ping needs a path there and a path home. |
| PC reaches its own LAN but nothing beyond | No default gateway set on the PC, or the wrong one. Check Desktop → IP Configuration before touching the routers. |
| OSPF neighbour never appears | Wildcard mask written as a subnet mask, mismatched area numbers, or the link is not up/up. Also allow 30-40 seconds for convergence before concluding it failed. |
| OSPF: routes missing for one network | That network has no matching "network" statement, or its interface is down. Check with "show ip protocols". |
| NAT: pings to outside fail, no errors | The "ip nat inside" and "ip nat outside" lines were never applied to the interfaces. This is the most commonly forgotten NAT step. |
| show ip nat translations is empty | Either NAT is not configured, or the entries timed out. Send fresh traffic and check again immediately. |
| IPv6 pings to gateway work, across link fail | Missing "ipv6 unicast-routing", or the static route was never added. The router will hold addresses and answer for itself without forwarding anything. |
| IPv6: gateway pings fine, far PC times out, tables look perfect | Check for a 2001:DB8 vs 2001:D88 mismatch between routers and PCs. Confirmed cause in lab — read every prefix character by character. |
| IPv6: one router has fewer route entries than the other | The one with fewer is missing its static route home. Both routers need 6 entries in this topology. |
| Serial reads down/down and the config looks correct | Run "show controllers serial 0/1/0". "No serial cable attached" means the port genuinely has no cable — it was never drawn, or landed on a different port. |
| "show controllers Serial0/1/0" rejected as invalid input | That form needs a space: "show controllers serial 0/1/0". |
| Corrected an IPv6 address but the old behaviour persists | IPv6 interfaces hold multiple addresses; "ipv6 address" adds rather than replaces. Use "no ipv6 address <old>" explicitly first. |
| All interfaces show "unassigned" in show ip interface brief | Normal in a pure-IPv6 build — that command only reports IPv4. Use "show ipv6 interface brief". |
| ACL blocks everything | The implicit "deny any any" at the end. Add an explicit "permit ip any any" as the final line. |
| ACL appears to do nothing | Line order — a broad permit above your deny makes the deny unreachable. Or it was applied in the wrong direction, or to the wrong interface. |
| Fixed an ACL line but behaviour unchanged | ACL lines append rather than overwrite. Remove the whole list with "no access-list 110" and retype it. |
| Everything worked, then broke after reload | Configuration was never saved. Finish every device with "write memory". |

# Command Quick Reference

| Purpose | Command |
| --- | --- |
| Enter privileged mode | enable |
| Enter global config | configure terminal |
| Discover interface names | show ip interface brief |
| Address an interface | interface <name> → ip address <ip> <mask> → no shutdown |
| Serial DCE timing | clock rate 64000 |
| Check DCE or DTE end | show controllers Serial0/0/0 |
| Static route | ip route <network> <mask> <next-hop> |
| Default route | ip route 0.0.0.0 0.0.0.0 <next-hop or interface> |
| Start OSPF | router ospf 1 |
| Advertise a network in OSPF | network <network> <wildcard> area 0 |
| Check OSPF adjacencies | show ip ospf neighbor |
| Mark NAT interfaces | ip nat inside / ip nat outside |
| Enable PAT | ip nat inside source list 1 interface <outside-int> overload |
| View NAT table | show ip nat translations |
| Enable IPv6 forwarding | ipv6 unicast-routing |
| Address an interface (IPv6) | ipv6 address <address>/<prefix> |
| IPv6 static route | ipv6 route <prefix>/<len> <next-hop> |
| IPv6 routing table | show ipv6 route |
| Extended ACL line | access-list 110 deny ip <src> <wildcard> <dst> <wildcard> |
| Apply an ACL | interface <name> → ip access-group 110 in |
| View ACLs and match counts | show access-lists |
| Delete an ACL | no access-list 110 |
| Routing table | show ip route |
| Save configuration | write memory |